ModelIt!

Student Activity Pack

Renewable Energy Grid Optimization: Powering the Future

Modeling the Complex Challenge of a 100 Percent Clean Energy Grid

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Grade Level: 9th Grade — Level 2: Advanced  |  Standard: HS-PS3-3, HS-ETS1-3

What's Inside

| Name: _______________________________ | Date: _______________ |
| --- | --- |

1. Pre-Assessment  -  What do you already know?

2. Scoring Rubric  -  How your work will be assessed

3. Component Analysis  -  Sort and classify system parts

4. Model Recording Page  -  Sketch your model

5. Simulation Observations  -  Record what happens

6. Research & Extend  -  Add new components

7. Reflection Questions  -  What did you learn?

8. STEM Challenge  -  Design a Regional Clean Energy Grid

Pre-Assessment: What Do You Know?

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer these questions BEFORE the lesson begins.

Why can't we just replace all power plants with solar panels?

What happens to electricity supply when the sun sets or the wind stops?

Draw a diagram showing how electricity gets from a solar panel to your home.

What are the pros and cons of different energy sources?

Scoring Rubric

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Use this rubric to check your own work!

| Skill | 4 - Expert | 3 - Proficient | 2 - Developing | 1 - Beginning |
| --- | --- | --- | --- | --- |
| Identifying Components | All correct + explains role | Most correct | Some correct | Struggles to identify |
| Establishing Relationships | All arrows correct + explains | Most correct | Some correct | Incorrect |
| Running Simulations | Tests all; predicts outcomes | Runs all; records | Runs some with guidance | Needs help |
| Explaining Results | Clear with evidence | Good explanation | Partial | Struggles |

Component Analysis

| Name: _______________________________ | Date: _______________ |
| --- | --- |

What is a Component?

A component is a part of a system. In our Renewable Energy Grid Optimization: Powering the Future system, we have these components:

*  Solar Output: Electricity generated from photovoltaic panels, varying with time of day, season, and cloud cover

*  Wind Output: Electricity generated from wind turbines, varying with wind speed and weather patterns

*  Battery Storage: Energy stored in lithium-ion or other battery systems that can be charged when supply exceeds demand

*  Grid Demand: Total electricity consumption by residential, commercial, and industrial users at any given time

*  Fossil Fuel Backup: Natural gas or other dispatchable generation used to fill gaps when renewables and storage cannot meet demand

*  Transmission Efficiency: The percentage of generated electricity that reaches consumers after transmission and distribution losses

*  Cost per kWh: The total cost of electricity including generation, storage, transmission, and backup costs

*  Carbon Emissions: Total CO2 and equivalent greenhouse gas emissions from the energy system

Sorting Activity

Sort into EXTERNAL (inputs from outside) and INTERNAL (processes inside the system):

| EXTERNAL Components (Come from outside) | INTERNAL Components (Happen inside system) |
| --- | --- |

Explain your reasoning:

Model Recording Page

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Sketch your model showing all components and their relationships.

Relationship Key:

(+) POSITIVE: When one increases, the other ALSO increases

(-) NEGATIVE: When one increases, the other DECREASES

My Model Sketch:

Draw components as boxes. Connect with arrows labeled + or -.

Explain ONE relationship in your model. Why does it work that way?

Simulation Observations

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Run each scenario in ModelIt! and record your observations.

Sunny Windy Day

Settings: Peak solar, strong wind, moderate demand

What happens to battery storage when supply exceeds demand? Where does the excess go?

Observation: What actually happened? Describe what you saw on the graph.

Worst Case

Settings: Zero solar, calm wind, peak demand

How long do batteries last? When does fossil backup kick in?

Observation: What actually happened? Describe what you saw on the graph.

Optimization

Settings: Vary mix to minimize cost and emissions

What percentage of renewables achieves the best balance of cost, reliability, and emissions?

Observation: What actually happened? Describe what you saw on the graph.

Research & Extend

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Real systems are more complex. EXTEND your model with new components!

Possible New Components:

*  Hydrogen Storage: Long-duration energy storage using excess renewable electricity to produce hydrogen via electrolysis

*  Nuclear Baseload: Constant-output nuclear power providing reliable baseload electricity independent of weather

*  Demand Response: Smart grid systems that shift flexible electricity demand to match renewable availability

My Extension:

New component I want to add: ________________________________

How does it connect? (Draw and explain)

What evidence supports these connections?

Reflection Questions

| Name: _______________________________ | Date: _______________ |
| --- | --- |

Answer after completing all activities.

Why is the last 20 percent of decarbonization so much harder than the first 80 percent?

What role does energy storage play in enabling higher renewable penetration?

How do trade-offs between cost, emissions, and reliability constrain grid design?

Why might geographic diversity of renewable installations reduce intermittency risk?

What technology breakthroughs would most change your optimal grid design?

STEM CHALLENGE: Design a Regional Clean Energy Grid

| Name: _______________________________ | Date: _______________ |
| --- | --- |

YOUR MISSION

Design the optimal energy mix for a region that achieves maximum renewable penetration while maintaining grid reliability and minimizing cost.

THE SCENARIO

A region of 5 million people must achieve 90 percent clean energy by 2035. Design the energy portfolio: how much solar, wind, storage, and backup do you need? What does it cost? How reliable is it during extreme weather?

ENGINEERING DESIGN THINKING

How would you distribute solar and wind installations geographically?

What types of storage would you deploy -- batteries, pumped hydro, hydrogen?

How would you handle seasonal variation in solar output?

What is the minimum fossil fuel backup needed to maintain 99.97 percent reliability?

MY DESIGN

Draw your design here:

EXPLAIN YOUR DESIGN

Why did you design it this way? Use scientific reasoning!